ENG 200 – Surveying

Math Review Problems

- Given the three angles shown, determine the total angle enclosed.
(answer:  8513’)

- A line bears 2314’ east of North.  A counter-clockwise angle of 7826’ is turned from that line to establish a second line.  What clockwise angle from North would coincide with the second line?
(answer: 30448’)

- A line extends 582.63’ along a bearing  3815’west of North.  How far west and how far north is the end point from the origin?
(answer: N 457.55’, W 360.70’)

- A surveyor is to extend the line from A to B, shown, then turn a 90 angle and proceed to C.  However, because of a swamp, she must stop 185.00’ short of B at point D.  She can turn a 40 angle and bypass the swamp.  How far should she follow that line to establish  point E along the line from B to C?  Determine the distance from B to E.
(answer: DE = 241.50’, BE = 155.23’)

- The surveyor in problem 4 really needs to set her transit over point B.  She turns the 40 angle, and measures along that line 185.00’ to establish point E (note: this point is not on line BC).  When she moves up to point E, what angle DEB and distance EB will bring her to point B?
(answer: DEB = 7000’,  BE = 126.55’)

- A surveyor follows the lines and angles shown, starting at A. How far, and in what direction is D from A?
(answer: 	AB: 	N = 375.00’	W = 0.00’
  		BC:	N = 137.19’	W = 169.42’
  		CD:	N = -57.64’	W = -49.23’
  		AD	N = 454.55’	W = 218.65’
  		AD = 504.40’ bearing 2541’19” W of N  )

- Point I is located 50.0’ N and 128.5’ W of point O.  Point J is located 189.8’ N and 38.0’ E of O.  How far N of O will a true N-S line intersect with a line connecting I and J? 
(answer: 157.89’)